Technical Report: SO-101 Robot Arm Project

End-to-end imitation learning on a physical 6-DoF manipulator

Dmitry  |  HuggingFace: TourEnduro  |  June 2026

# 1. Project Overview

This project covers the full pipeline from platform selection and hardware procurement to autonomous pick-and-place task execution using a trained neural network. The SO-101 platform (Hugging Face LeRobot) was used with a leader-follower teleoperation architecture for dataset collection.

Key outcomes:

- Assembled two robot arms from 3D-printed parts and off-the-shelf electronics

- Implemented leader-follower teleoperation system for data collection

- Recorded 50-episode pick-and-place dataset

- Trained an ACT (Action Chunking with Transformers) imitation learning policy

- Deployed the trained model for autonomous task execution on the physical robot

# 2. Platform Selection and Components

## 2.1 SO-101 vs PAROL6

Two platforms were evaluated:

- SO-101 (Hugging Face LeRobot): ~250-300 EUR, direct Python/AI stack integration, active community

- PAROL6 (Source Robotics): real total cost ~1,450-1,500 EUR including planetary gearboxes and control board

Decision: SO-101 as the optimal choice for a first robotics portfolio project — lower cost, faster iteration, and direct integration with modern ML tooling.

## 2.2 Bill of Materials

| Component | Qty | Source |
| --- | --- | --- |
| STS3215 7.4V 1/345 (C001) — follower motors | 7 | Alibaba |
| STS3215 7.4V 1/191 (C044) — leader joints 1, 3 | 2 | Alibaba |
| STS3215 7.4V 1/147 (C046) — leader joints 4, 5, 6 | 3 | Alibaba |
| Waveshare Bus Servo Adapter (A) | 2 | Amazon |
| 5V 4A Power Supply | 2 | Amazon |
| Innomaker U20CAM 1080P UVC Camera | 1 | Amazon |
| USB-C Cables | 2 | Amazon |
| Precision Screwdriver Set | 1 | Amazon |
| Table Clamps 100x50mm | 2 | Amazon |

Note: The LEICKE 5V power supply with 5.5x2.5mm connector is compatible with the Waveshare board despite the 0.4mm size difference due to a spring-loaded center pin.

# 3. Software Environment Setup

## 3.1 WSL2 Ubuntu + Conda (initial attempt)

LeRobot was initially installed on WSL2 Ubuntu 22.04 with Miniconda. USB devices were passed through using usbipd-win.

Issues encountered with WSL2:

- USB passthrough via usbipd is unstable under intensive bus polling — sync_read fails with 'No status packet' errors

- Motor calibration consistently interrupted due to packet loss

- Keyboard arrow keys had no effect during recording (pynput module missing)

Note: Resolution: switched to native Windows + Miniconda for Windows. COM ports are accessed directly without usbipd.

## 3.2 Native Windows (final configuration)

Miniconda for Windows was installed and a dedicated environment created:

conda create -n lerobot python=3.12

conda activate lerobot

git clone https://github.com/huggingface/lerobot.git

cd lerobot

pip install -e ".[feetech,dataset]"

pip install pynput

Note: LeRobot requires Python 3.12+ (changed from 3.10 during the project). On Windows, COM ports are used directly without any USB passthrough layer.

# 4. Motor Configuration

## 4.1 Identifying COM Ports

lerobot-find-port

Result: COM7 = Follower arm, COM8 = Leader arm

## 4.2 Assigning Motor IDs

Each motor is connected to the board individually and assigned a unique ID. Order: gripper (ID=6) down to shoulder_pan (ID=1).

Follower (all 6 motors identical, C001):

lerobot-setup-motors --robot.type=so101_follower --robot.port=COM7

Leader (different gear ratios per joint):

lerobot-setup-motors --teleop.type=so101_leader --teleop.port=COM8

## 4.3 Leader Motor Assignment

| ID | Joint | Motor | Gear Ratio |
| --- | --- | --- | --- |
| 1 | Base / Shoulder Pan | C044 | 1/191 |
| 2 | Shoulder Lift | C001 | 1/345 |
| 3 | Elbow Flex | C044 | 1/191 |
| 4 | Wrist Flex | C046 | 1/147 |
| 5 | Wrist Roll | C046 | 1/147 |
| 6 | Gripper | C046 | 1/147 |

# 5. Calibration

## 5.1 Follower Calibration

lerobot-calibrate --robot.type=so101_follower --robot.port=COM7 --robot.id=my_follower_arm

Procedure: move arm to mid-range position -> press Enter -> move each joint (except wrist_roll) through its full range of motion -> press Enter.

Calibration file saved to: C:\Users\user\.cache\huggingface\lerobot\calibration\robots\so_follower\my_follower_arm.json

Note: wrist_roll is intentionally excluded from range calibration — it rotates 360 degrees continuously with no fixed min/max.

## 5.2 Leader Calibration

lerobot-calibrate --teleop.type=so101_leader --teleop.port=COM8 --teleop.id=my_leader_arm

# 6. Teleoperation

## 6.1 Basic Teleoperation

lerobot-teleoperate --robot.type=so101_follower --robot.port=COM7 --robot.id=my_follower_arm --teleop.type=so101_leader --teleop.port=COM8 --teleop.id=my_leader_arm

## 6.2 Teleoperation with Camera

The Innomaker U20CAM camera is mounted on the wrist of the follower arm. Camera index detection:

python -c "import cv2; [print(i) for i in range(5) if cv2.VideoCapture(i).isOpened()]"

Result: index 0 = built-in laptop webcam, index 1 = wrist-mounted UVC camera.

lerobot-teleoperate --robot.type=so101_follower --robot.port=COM7 --robot.id=my_follower_arm --teleop.type=so101_leader --teleop.port=COM8 --teleop.id=my_leader_arm --robot.cameras={"wrist":{"type":"opencv","index_or_path":1,"width":640,"height":480,"fps":30}}

Note: Connect the camera to a separate USB port from the motor control boards to avoid USB bandwidth conflicts.

## 6.3 Keyboard Controls During Recording

- Right arrow (->): stop current episode early, move to next

- Left arrow (<-): discard current episode, re-record it

- ESC: stop entire session and save dataset

Note: pynput must be installed for keyboard controls to work: pip install pynput

# 7. Dataset Collection

## 7.1 Recording Parameters

Task: pick-and-place a cube from position A to position B.

- 50 episodes total

- 30 seconds per episode (--dataset.episode_time_s=30)

- 10 seconds reset time (--dataset.reset_time_s=10)

- Total recording time: ~40 minutes

- Final dataset size: 355 MB

## 7.2 Record Command

lerobot-record --robot.type=so101_follower --robot.port=COM7 --robot.id=my_follower_arm \

--teleop.type=so101_leader --teleop.port=COM8 --teleop.id=my_leader_arm \

--robot.cameras={"wrist":{"type":"opencv","index_or_path":1,"width":640,"height":480,"fps":30}} \

--dataset.repo_id=my_dataset --dataset.num_episodes=50 \

--dataset.single_task="Pick the cube and place it on the other side" \

--dataset.episode_time_s=30 --dataset.reset_time_s=10

## 7.3 Resuming Interrupted Recording

If recording is interrupted (e.g. camera cable disconnected), resume with --resume=true. Important: set --dataset.num_episodes to the REMAINING episode count, not the total.

lerobot-record ... --dataset.num_episodes=<REMAINING> --resume=true \

--dataset.root="C:\Users\user\.cache\huggingface\lerobot\my_dataset_<TIMESTAMP>"

## 7.4 Uploading Dataset to Hugging Face Hub

Upload the full dataset folder preserving the data/meta/videos structure:

python -c "from huggingface_hub import HfApi; api = HfApi(); api.upload_folder(folder_path=r'PATH', repo_id='TourEnduro/so101_pick_place', repo_type='dataset')"

Published dataset: https://huggingface.co/datasets/TourEnduro/so101_pick_place

Note: Upload the entire dataset folder to the repository root — the data/, meta/, and videos/ directories must be at the top level, not nested inside a subfolder.

# 8. Model Training

## 8.1 Infrastructure

Local GPU (Quadro K3000M, 2012, 2GB VRAM) is not supported by modern PyTorch. Training was performed on Google Colab using a T4 GPU.

Training notebook: https://huggingface.co/docs/lerobot/notebooks#training-act

## 8.2 Training Command

lerobot-train \

--dataset.repo_id=TourEnduro/so101_pick_place \

--dataset.revision=main \

--policy.type=act \

--output_dir=outputs/train/hf_act_record0 \

--policy.device=cuda \

--wandb.enable=False \

--policy.repo_id=TourEnduro/so101_act_policy \

--batch_size=8 \

--steps=20000

Note: --dataset.revision=main is required — without it LeRobot cannot locate the dataset on the Hub.

## 8.3 Training Results

- Training time: ~97 minutes on T4 GPU

- Final loss: 0.141

- Training speed: ~3.4 steps/sec

- Published model: https://huggingface.co/TourEnduro/so101_act_policy

# 9. Inference on Physical Robot

## 9.1 Config Compatibility Fix

After updating LeRobot, a backward compatibility issue appeared: the pretrained_revision field in the model's config.json was no longer accepted by the new ACTConfig. Fix: remove the field from config.json and re-upload to the Hub.

# Open config.json in a text editor and delete these two lines:

# "pretrained_path": null,

# "pretrained_revision": null,

# File path:

# C:\Users\user\.cache\huggingface\hub\models--TourEnduro--so101_act_policy\snapshots\<HASH>\config.json

Re-upload the fixed config:

python -c "from huggingface_hub import HfApi; api = HfApi(); api.upload_file(path_or_fileobj=r'PATH\config.json', path_in_repo='config.json', repo_id='TourEnduro/so101_act_policy', repo_type='model')"

## 9.2 Inference Command

In the current version of LeRobot, lerobot-rollout is used for inference on a real robot (not lerobot-record):

C:\Users\user\miniconda3\envs\lerobot\python.exe -m lerobot.scripts.lerobot_rollout \

--robot.type=so101_follower \

--robot.port=COM7 \

--robot.id=my_follower_arm \

--robot.cameras={"wrist":{"type":"opencv","index_or_path":1,"width":640,"height":480,"fps":30}} \

--policy.path=TourEnduro/so101_act_policy

## 9.3 Results

The model successfully executes the pick-and-place task autonomously — 3 successful attempts in initial runs. Motion is imprecise, which is expected for a model trained on 50 episodes with a single wrist camera. Performance can be improved with more training data and additional camera viewpoints.

# 10. Potential Improvements

- Increase dataset to 100-200 episodes with greater variation in object placement

- Add a second fixed camera (side view) to improve spatial understanding

- Increase training steps to 50,000+

- Build URDF model for ROS2 / MoveIt2 integration

- Implement ROS2 nodes for joint state publishing and trajectory execution

# 11. Resources

- Dataset: https://huggingface.co/datasets/TourEnduro/so101_pick_place

- Trained Policy: https://huggingface.co/TourEnduro/so101_act_policy

- HuggingFace Profile: https://huggingface.co/TourEnduro

- LeRobot Documentation: https://huggingface.co/docs/lerobot/so101

- SO-ARM100 GitHub: https://github.com/TheRobotStudio/SO-ARM100